CS/ECE 252: INTRODUCTION TO COMPUTER ENGINEERING

UNIVERSITY OF WISCONSIN—MADISON

Prof. Gurindar Sohi

TAs: Rebecca Lam and Newsha Ardalani

Midterm Examination 1

In Class (50 minutes)

Wednesday, September 28, 2011

Weight: 17.5%

NO: BOOK(S), NOTE(S), OR CALCULATORS OF ANY SORT.

The exam has nine pages.  Circle your final answers. Plan your time carefully since some problems are longer than others.  Must turn in the pages 1-7.

LAST NAME:	___________________________________________________________

FIRST NAME: 	___________________________________________________________

SECTION: 	___________________________________________________________

ID# 		___________________________________________________________

| Problem | Maximum Points | Points Earned |
| --- | --- | --- |
| 1 | 1 |  |
| 2 | 2 |  |
| 3 | 2 |  |
| 4 | 4 |  |
| 5 | 1 |  |
| 6 | 1 |  |
| 7 | 4 |  |
| 8 | 4 |  |
| 9 | 2 |  |
| 10 | 2 |  |
| 11 | 3 |  |
| 12 | 4 |  |
| Total | 30 |  |

Problem 1:	Convert the following binary sequence to hexadecimal: 0111010011100110101111. (1 Point)

0x1D39AF

Problem 2:	Convert the following unsigned fixed point binary fraction into decimal (base 10) form: 101101.01. (2 Points)

101101 = 45

0.01 = ¼

45.25

Problem 3:	Convert the following decimal number into its fixed point binary fraction form: 12.50. (2 Points)

12 = 1100

0.5 = ½ = 0.1

Problem 4:	Show the result of performing the following logic operations on X = 0110 and Y = 1101:

- X AND Y	(1 Point)

0110 AND 1101 = 0100

- X OR Y	(1 Point)

0110 OR 1101 = 1111

- (NOT X) OR (X AND Y)	(2 Points)

~X = 1001	X AND Y = 0100	1001 OR 0100 = 1101

Problem 5:	Convert the null terminated ASCII string “.org” to its binary. Do not include the quotes and use 8-bits per character (see attached ASCII table) (1 Point):

2E 6F 72 67 00 = 0010 1110 0110 1111 0111 0010 0110 0111 0000 0000

Problem 6:	Convert the following binary code into an ASCII string (see attached ASCII table): 01100101 01100011 01000101 00000000:  (1 Point)

ecEnul

Problem 7:	Consider the following 32-bit IEEE floating point number: 11000001010100000000000000000000: (4 Points)

a) Fill in the table below showing the bits for each field

| Sign Bit (1 bit) | Exponent (8 Bits) | Fraction (23 Bits) |
| --- | --- | --- |
| 1 | 1000 0010 | 10100000000000000000000 |

b) What is the decimal value of the above IEEE floating point number:

1000 0010 = 130

130 – 127 = 3

-1.101 23 = -1101 = -13

Problem 8: a) Add the following two 4-bit 2’s complement binary numbers: 1011+1010: Is the output the correct answer? Explain why or why not.	 (2 Point)

1011 = -5

+1010 = -6

0101 = 5

Overflow

b) Subtract the following two 4-bit 2’s complement binary numbers: 1011-1100: Is the output the correct answer? Explain why or why not. (2 Points)

1011

-1100

Take the complement of 1100: 1100 becomes 0011 + 0001 = 0100

1011

+0100

1111

Problem 9:	The following questions refer to a 6-bit binary number. Give all answers in decimal (base 10): (2 Points)

- What is the largest 2’s complement number that can be represented:

2(6-1) – 1 = 31

- How many representations for zero does a 2’s complement number have:

1

- How many representations for zero does a 1’s complement number have:

2

- What is the largest value an unsigned 6-bit number can represent:

26 – 1 = 63

Problem 10: Put the following items/terms in order according to their level of abstraction relative to one another, label the most abstract 1 and least abstract 5 according to their hierarchy: (2 Points)

| 3 | Assembly/Binary Code |
| --- | --- |
| 4 | Microprocessor design |
| 1 | Algorithm to solve a problem |
| 5 | Transistor and logic layout |
| 2 | C/C++ Code |

Problem 11: In the table below, fill in the number of the concept (1 through 6) that best matches the corresponding statement:  (3 Points)

- Universal Turing Machine

- Definiteness

- Effective Computability

- Finiteness

- Abstraction

- Language/Code

| 3 | Determines whether or not a problem is solvable |
| --- | --- |
| 5 | Underlying mechanisms are hidden or unknown |
| 1 | A device that that can accept data and instructions |
| 2 | Each step of a process must be clearly laid out |
| 6 | Can be used to write an algorithm that a computer can understand |
| 4 | Will not run on forever, will stop at some point |

Problem 12: In class we talked about logical operations on sets of bits (like question 4 above).  Such logical operations can be carried out on n-bit variables, X and Y.

Your job is to carry out an operation whose result will be zero, 00 (in hex), when an upper case letter is typed on the keyboard whose result is a non-zero value when a lower case letter is typed.

Use the variable X to denote the 8-bit input value from the keyboard. You will need to determine the 8-bit variable Y, and the logical operation that you will need to carry out.  Show your work. (4 Points)

Hint: ‘A’ = 41 (in hex) = 0100 0001 and ‘a’ = 61 (in hex) = 0110 0001

Output = X AND Y, where Y = 0x20

0100 0001 = ‘A’			0110 0001 = ‘a’

0010 0000 = 0x20		0010 0000 = 0x20

0000 0000 = 0x00		0010 0000 ≠ 0x00

ASCII Table

| Character | Hex | Character | Hex | Character | Hex | Character | Hex |
| --- | --- | --- | --- | --- | --- | --- | --- |
| nul | 00 | sp | 20 | @ | 40 | ` | 60 |
| soh | 01 | ! | 21 | A | 41 | a | 61 |
| stx | 02 | “ | 22 | B | 42 | b | 62 |
| etx | 03 | # | 23 | C | 43 | c | 63 |
| eot | 04 | $ | 24 | D | 44 | d | 64 |
| enq | 05 | % | 25 | E | 45 | e | 65 |
| ack | 06 | & | 26 | F | 46 | f | 66 |
| bel | 07 | ‘ (Apostr.) | 27 | G | 47 | g | 67 |
| bs | 08 | ( | 28 | H | 48 | h | 68 |
| ht | 09 | ) | 29 | I | 49 | i | 69 |
| lf | 0A | * | 2A | J | 4A | j | 6A |
| vt | 0B | + | 2B | K | 4B | k | 6B |
| ff | 0C | , (Comma) | 2C | L | 4C | l | 6C |
| cr | 0D | - | 2D | M | 4D | m | 6D |
| so | 0E | . (Period) | 2E | N | 4E | n | 6E |
| si | 0F | / | 2F | O | 4F | o | 6F |
| dle | 10 | 0 | 30 | P | 50 | p | 70 |
| dc1 | 11 | 1 | 31 | Q | 51 | q | 71 |
| dc2 | 12 | 2 | 32 | R | 52 | r | 72 |
| dc3 | 13 | 3 | 33 | S | 53 | s | 73 |
| dc4 | 14 | 4 | 34 | T | 54 | t | 74 |
| nak | 15 | 5 | 35 | U | 55 | u | 75 |
| syn | 16 | 6 | 36 | V | 56 | v | 76 |
| etb | 17 | 7 | 37 | W | 57 | w | 77 |
| can | 18 | 8 | 38 | X | 58 | x | 78 |
| em | 19 | 9 | 39 | Y | 59 | y | 79 |
| sub | 1A | : | 3A | Z | 5A | z | 7A |
| esc | 1B | ; | 3B | [ | 5B | { | 7B |
| fs | 1C | < | 3C | \ | 5C | ¦ | 7C |
| gs | 1D | = | 3D | ] | 5D | } | 7D |
| rs | 1E | > | 3E | ^ | 5E | ~ | 7E |
| us | 1F | ? | 3F | _ (Undrscre) | 5F | del | 7F |

Extra page for hand written work, if needed. This page is not required and will NOT affect your grade. You don’t even need to hand this page in.