CS/ECE 252: INTRODUCTION TO COMPUTER ENGINEERING

UNIVERSITY OF WISCONSIN—MADISON

Prof. Gurindar Sohi

TAs: Pradip Vallathol and Junaid Khalid

Examination 4

In Class (50 minutes)

Wednesday, December 12, 2012

Weight: 17.5%

NO: BOOK(S), NOTE(S), OR CALCULATORS OF ANY SORT.

The exam has nine pages.  Circle your final answers. Plan your time carefully since some problems are longer than others.  You must turn in the pages 1-7.

LAST NAME:	___________________________________________________________

FIRST NAME: 	___________________________________________________________

ID# 		___________________________________________________________

| Problem | Maximum Points | Points Earned |
| --- | --- | --- |
| 1 | 6 |  |
| 2 | 6 |  |
| 3 | 6 |  |
| 4 | 3 |  |
| 5 | 3 |  |
| 6 | 6 |  |
| Total | 30 |  |

Problem 1:	Assembly Language

- Briefly explain the four assembly errors in the following LC-3 program.												(4 Points)

.ORIG x3000

LD  R2, INPUT

AND R0, R0, #0

ADD R1, R0, #1

BR NEXT

LOOP	  AND R4, R2, R1

BRz SKIP

OR  R0, R0, #1

SKIP	  ADD R1, R1, R1

ADD R3, R3, x2F

LD  R6, SKIP

NOT R6, R6

BRzp LOOP

INPUT	  .FILL x1997

SKIP	  .FILL x1998

.END

- Which of the following (if any) of the following pseudo-ops can be used multiple times in a single assembly file. Circle all options that apply.		(2 Points)

  - .ORIG

  - .FILL

  - .BLKW

  - .STRINGZ

  - .END

Problem 2:	Two pass Assembly Process

An LC-3 assembly language program in given below:

.ORIG x3000

AND R3, R3, #0

LD  R0, M0

LD  R1, M1

LD  R2, M2

LOOP	   ADD R3, R3, #1

ADD R3, R3, R2

ADD R0, R0, #-1

BRn LOOP

DONE    ST  R3, RESULT

HALT

RESULT  .FILL x0000

M0      .BLKW #5

M1      .STRINGZ “CS-ECE-252”

M2      .FILL x0009

.END

- A symbol table is created during the first pass by the assembler. Fill in the following symbol table for the above program:			(4 Points)

| Symbol | Address |
| --- | --- |

- The assembly program is converted into a binary file during the second pass by the assembler. Fill in the binary instructions at the following memory locations:										(2 Points)

| Address | Instructions |
| --- | --- |
| x3001 |  |
| x3007 |  |

Problem 3:	Traps and Subroutines					(6 Points)

The following LC-3 assembly program takes a single character as input from the user. If the input character is a digit (0-9), it prints the message “Is a digit” on the display. This process is continued until the user enters the termination character ‘#’, and the program halts. Fill in the missing parts of the program indicated by ________.

.ORIG x3000

GETINPUT	TRAP ______		; Input a character from the user

; (Do not echo it on the display)

LD  R1, TERMCHAR	; termination check

ADD R1, R0, R1

___________		; Branch to END on ‘#’

_______________  	; Call CHECKINPUT subroutine

BR  GETINPUT

END		HALT

CHECKINPUT

________, SAVELOC	; Save something here

LD  R2, DIGIT0

ADD R2, R0, R2

BRn RELOAD

LD  R2, DIGIT9

ADD R2, R0, R2

BRp RELOAD

DISP_IS	______________	; print a string

TRAP ______		; to the display

RELOAD	________, SAVELOC	; Load something here

RET

; Data

SAVELOC	.BLKW		#1

STR_IS	.STRINGZ	"Is a digit\n"

STR_NOT	.STRINGZ	"Not a Digit\n"

TERMCHAR	.FILL		0xFFDD	; negative ASCII value of ‘#’

DIGIT0	.FILL		0xFFD0	; negative ASCII value of ‘0’

DIGIT9	.FILL		0xFFC7	; negative ASCII value of ‘9’

.END

Problem 4:	I/O

- Briefly explain the difference between interrupt-driven I/O and polling based I/O?									(2 Points)

- What is the main reason to prefer asynchronous I/O over synchronous I/O in recent microprocessor designs?					(1 Point)

Problem 5: Trap Handling							(3 Points)

List the main steps of the TRAP mechanism involved in executing the instruction TRAP x67.

Problem 6:	Short Answer Questions

Answer the flowing questions briefly.

- What important feature does the instruction JSRR provide that JSR does not?										(1 Point)

- Explain briefly the problem that the callee-save and the caller-save approaches are trying to solve.							(2 Point)

- How many trap service routines can be defined in LC-3?		(1 Point)

- What is the use of Comments in an assembly program?		(1 Point)

- What happens during the linking phase of an assembly program?	(1 Point)

Extra page for hand written work, if needed. This page is not required and will NOT affect your grade. You don’t even need to hand this page in.

ASCII Table

| Character | Hex | Character | Hex | Character | Hex | Character | Hex |
| --- | --- | --- | --- | --- | --- | --- | --- |
| nul | 00 | sp | 20 | @ | 40 | ` | 60 |
| soh | 01 | ! | 21 | A | 41 | a | 61 |
| stx | 02 | “ | 22 | B | 42 | b | 62 |
| etx | 03 | # | 23 | C | 43 | c | 63 |
| eot | 04 | $ | 24 | D | 44 | d | 64 |
| enq | 05 | % | 25 | E | 45 | e | 65 |
| ack | 06 | & | 26 | F | 46 | f | 66 |
| bel | 07 | ‘ (Apostr.) | 27 | G | 47 | g | 67 |
| bs | 08 | ( | 28 | H | 48 | h | 68 |
| ht | 09 | ) | 29 | I | 49 | i | 69 |
| lf | 0A | * | 2A | J | 4A | j | 6A |
| vt | 0B | + | 2B | K | 4B | k | 6B |
| ff | 0C | , (Comma) | 2C | L | 4C | l | 6C |
| cr | 0D | - | 2D | M | 4D | m | 6D |
| so | 0E | . (Period) | 2E | N | 4E | n | 6E |
| si | 0F | / | 2F | O | 4F | o | 6F |
| dle | 10 | 0 | 30 | P | 50 | p | 70 |
| dc1 | 11 | 1 | 31 | Q | 51 | q | 71 |
| dc2 | 12 | 2 | 32 | R | 52 | r | 72 |
| dc3 | 13 | 3 | 33 | S | 53 | s | 73 |
| dc4 | 14 | 4 | 34 | T | 54 | t | 74 |
| nak | 15 | 5 | 35 | U | 55 | u | 75 |
| syn | 16 | 6 | 36 | V | 56 | v | 76 |
| etb | 17 | 7 | 37 | W | 57 | w | 77 |
| can | 18 | 8 | 38 | X | 58 | x | 78 |
| em | 19 | 9 | 39 | Y | 59 | y | 79 |
| sub | 1A | : | 3A | Z | 5A | z | 7A |
| esc | 1B | ; | 3B | [ | 5B | { | 7B |
| fs | 1C | < | 3C | \ | 5C | ¦ | 7C |
| gs | 1D | = | 3D | ] | 5D | } | 7D |
| rs | 1E | > | 3E | ^ | 5E | ~ | 7E |
| us | 1F | ? | 3F | _ (Undrscre) | 5F | del | 7F |

LC-3 Instruction Set (Entered by Mark D. Hill on 03/14/2007; last update 03/15/2007)

PC’: incremented PC. setcc(): set condition codes N, Z, and P. mem[A]:memory contents at address A.
SEXT(immediate): sign-extend immediate to 16 bits. ZEXT(immediate): zero-extend immediate to 16 bits.

15  14  13  12  11  10   9   8   7   6   5   4   3   2   1   0

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ ADD DR, SR1, SR2 ; Addition

| 0   0   0   1 |    DR     |    SR1    | 0 | 0   0 |    SR2    |

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ DR  SR1 + SR2 also setcc()

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ ADD DR, SR1, imm5 ; Addition with Immediate

| 0   0   0   1 |    DR     |    SR1    | 1 |       imm5        |

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ DR  SR1 + SEXT(imm5) also setcc()

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ AND  DR, SR1, SR2 ; Bit-wise AND

| 0   1   0   1 |    DR     |    SR1    | 0 | 0   0 |    SR2    |

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ DR  SR1 AND SR2 also setcc()

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ AND DR,SR1,imm5 ; Bit-wise AND with Immediate

| 0   1   0   1 |    DR     |    SR1    | 1 |       imm5        |

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ DR  SR1 AND SEXT(imm5) also setcc()

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ BRx,label (where x={n,z,p,zp,np,nz,nzp}); Branch

| 0   0   0   0 | n | z | p |              PCoffset9            | GO  ((n and N) OR (z AND Z) OR (p AND P))

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ if(GO is true) then PCPC’+ SEXT(PCoffset9)

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ JMP BaseR ; Jump

| 1   1   0   0 | 0   0   0 |   BaseR   | 0   0   0   0   0   0 |

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ PC  BaseR

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ JSR label ; Jump to Subroutine

| 0   1   0   0 | 1 |                  PCoffset11               |

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ R7  PC’, PC  PC’ + SEXT(PCoffset11)

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ JSRR BaseR ; Jump to Subroutine in Register

| 0   1   0   0 | 0 | 0   0 |   BaseR   | 0   0   0   0   0   0 |

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ temp  PC’, PC  BaseR, R7  temp

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ LD DR, label ; Load PC-Relative

| 0   0   1   0 |    DR     |              PCoffset9            |

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ DR  mem[PC’ + SEXT(PCoffset9)] also setcc()

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ LDI DR, label ; Load Indirect

| 1   0   1   0 |    DR     |              PCoffset9            |

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ DRmem[mem[PC’+SEXT(PCoffset9)]] also setcc()

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ LDR DR, BaseR, offset6 ; Load Base+Offset

| 0   1   1   0 |    DR     |   BaseR   |        offset6        |

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ DR  mem[BaseR + SEXT(offset6)] also setcc()

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ LEA, DR, label ; Load Effective Address

| 1   1   1   0 |    DR     |              PCoffset9            |

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ DR  PC’ + SEXT(PCoffset9) also setcc()

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ NOT DR, SR ; Bit-wise Complement

| 1   0   0   1 |    DR     |     SR    | 1 | 1   1   1   1   1 |

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ DR  NOT(SR) also setcc()

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ RET ; Return from Subroutine

| 1   1   0   0 | 0   0   0 | 1   1   1 | 0   0   0   0   0   0 |

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ PC  R7

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ RTI ; Return from Interrupt

| 1   0   0   0 | 0   0   0   0   0   0   0   0   0   0   0   0 |

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ See textbook (2nd Ed. page 537).

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ ST SR, label ; Store PC-Relative

| 0   0   1   1 |    SR     |              PCoffset9            |

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ mem[PC’ + SEXT(PCoffset9)]  SR

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ STI, SR, label ; Store Indirect

| 1   0   1   1 |    SR     |              PCoffset9            |

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ mem[mem[PC’ + SEXT(PCoffset9)]]  SR

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ STR SR, BaseR, offset6 ; Store Base+Offset

| 0   1   1   1 |    SR     |   BaseR   |        offset6        |

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ mem[BaseR + SEXT(offset6)]  SR

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ TRAP ; System Call

| 1   1   1   1 | 0   0   0   0 |           trapvect8           |

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ R7  PC’, PC  mem[ZEXT(trapvect8)]

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ ; Unused Opcode

| 1   1   0   1 |                                               |

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+ Initiate illegal opcode exception

15  14  13  12  11  10   9   8   7   6   5   4   3   2   1   0

TRAP CODES